SAFETY ALERT

The below information has been issued on behalf of the EFFC

to make others aware of the potential risks and

possible precautions to take in order to avoid these.

| Date/Time of Incident: | May 2024 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Incident: | Exclusion and Restricted Zone Breach |  |  |  |  |  |  |
| Nature of Incident/Injury: | Operatives working inside defined exclusion and restricted zones |  |  |  |  |  |  |
| Details of Incident: | We have recently had two incidents that involve operatives working within a lifting operation exclusion zone and operatives working within a piling restricted zone. Both working activities were NOT managed correctly in accordance with company procedures, task brief, risk assessments and lift plans. |  |  |  |  |  |  |
| Root cause (if known): | Operatives stood within the exclusion zone whilst lifting operations were taking place. Non-working operatives stood within the restricted zone whilst piling. |  |  |  |  |  |  |
| Action Taken: | Reinforcement of exclusion and restricted zone definitions and control measures. Promotion of FPS Restricted Zone Handbook and guidance. Rebriefing of roles and responsibilities, including: - Rig/Plant Operator: Must not move plant without direction and must give/receive visual confirmation ('Thumbs up') before entry. - Banksperson: In control of all rig/plant movement; must confirm entry is safe with operator. - Authorised Person: Only permitted individuals may enter restricted zones. Reissue of the FPS Restricted Zone Handbook available at: www.fps.org.uk/content/uploads/2021/02/FPS-RESTRICTED-ZONE-HANDBOOK_A6_14029.pdf |  |  |  |  |  |  |
| Lessons Learnt: | Exclusion zones must be clearly defined, communicated, and enforced to keep workers safe from hazards such as lifting operations, piling, and moving equipment. Restricted zones must be detailed in RAMS and respected, with only authorised personnel granted access. Strict adherence to roles and visual communication protocols is critical for ensuring safety within these zones. |  |  |  |  |  |  |